Storm Water Pollution Prevention Plan

for the

General Industrial Permit

(Ohio EPA Permit No.:  OHR000004)

at the

Former Satralloy Site

4243 County Road 74

Mingo Junction, Jefferson County, Ohio  43938

Prepared for:

Cyprus Amax Minerals Company

c/o Gallagher & Kennedy PA

2575 East Camelback Road

Phoenix, Arizona  85016

Prepared By:

GT Environmental, Inc.

635 Park Meadow Road, Suite 112

Westerville, Ohio 43081

and

Golder Associates Inc.

18300 NE Union Hill Road, Suite 200

Redmond, Washington  98052  USA

November 1, 2010

Storm Water Pollution Prevention Plan (SWPPP)

Plan Certification

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified associates properly gathered and evaluated the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information is, to the best of my knowledge and belief, true, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

_____________________________________

Name

_____________________________________

Title / Cyprus Amax Authorized Representative

_____________________________________

Signature

_____________________________________

Date

Storm Water Pollution Prevention Plan (SWPPP)

Appointment of Duly Authorized Representative

For the purposes of managing the Storm Water Pollution Prevention Plan and its associated inspections and activities, the person serving as Cyprus Amax Project Manager (CAPM), as specified in this SWPPP, is appointed as the Duly Authorized Representative.  The CAPM has the responsibility for the overall operation of the Satralloy Site.

_____________________________________

Name

_____________________________________

Responsible Corporate Officer / Title

_____________________________________

Signature

_____________________________________

Date

Table of Contents

1.0	Introduction	1

1.1	Site Description	1

1.2	NPDES General Industrial Storm Water Permit	2

1.3	Pollution Prevention Team	2

1.4	Regulatory and Emergency Contact Information	3

2.0	description of potential pollutant sources	4

2.1	Drainage	4

2.2	Inventory of Exposed Materials	4

2.3	Spills and Leaks	5

2.4	Sampling Data	5

2.5	Risk Identification and Summary of Potential Pollutant Sources	5

3.0	measures and controls	6

3.1	Erosion and Sediment Controls and Management of Runoff	6

3.2	Good Housekeeping	6

3.3	Preventative Maintenance	6

3.4	Spill Prevention and Response	6

3.5	Inspections	7

3.5.1	Site Inspection Frequency	7

3.5.2	Site Inspection Documentation	7

3.5.3	SWPPP Activities Log	7

3.6	Training	8

3.7	Log of Changes to the SWPPP	8

3.8	Maintaining and Updating the SWPPP	8

3.9	Recordkeeping and Internal Reporting Procedures	8

3.9.1	Records Retention	8

3.9.2	Access to Plans and Records	8

3.9.3	Non-Storm water Discharges	8

4.0	Plan compliance	9

4.1	Comprehensive Site Compliance Evaluation	9

4.2	Consistency with Other Plans	9

4.3	Additional Requirements for “Section 313 Water Priority Chemicals”	9

4.4	Additional Requirements for Discharges to MS4s	9

5.0	STORM WATER QUALITY MONITORING	10

5.1	Sample Analyses	10

5.2	Storm Water Drainage and Sample Locations	10

5.2.1	Drainage Areas	10

5.2.2	Discharge Locations	11

5.2.3	Sample Locations	11

List of Tables

Table 1	Pollution Prevention Team Members

Table 2	Regulatory and Emergency Contacts

List of Figures

1	Site Location

2  	Site Map

3	Site Drainage and Sampling Locations

List of Appendices

Appendix A	NPDES Permit

Appendix B	NOI and Ohio EPA Acknowledgment Letter

Appendix C	Satralloy Site Hydrologic Analysis Tech Memo

Appendix D	Inspection Form and Activities Log

Appendix E	Training Log

Appendix F	SWPPP Amendment Log

Appendix G	Annual Site Compliance Evaluation Form

Appendix H	Storm Water Discharge Areas and Sampling Locations

# Introduction

This Storm Water Pollution Prevention Plan (SWPPP) has been prepared on behalf of Cyprus Amax Minerals Company ("Cyprus Amax" or "Owner") for the Former Satralloy Site (the Site) in Jefferson County, Ohio.

Construction activities, consisting of abatement of regulated materials, building demolition, and removal of ancillary structures located in the Lowlands Area, will be managed under the General Construction Storm Water Permit # OHC000003.  Upon completion of the construction activities, the storm water in the Construction Areas will be managed under this Industrial SWPPP.

## Site Description

The Former Satralloy site consists of approximately 333 acres of land that includes an abandoned ferrochromium-alloy plant.  The Site is located in Cross Creek Township, Jefferson County, Ohio, approximately 4 miles south of Steubenville.  Site access is via County Road 74 (Gould Road).  The Site is bordered on the north, west, and south by Cross Creek, a perennial stream that empties into the Ohio River.  The site location is shown on Figure 1.

The site is currently a non-operating facility, which was used for industrial operations, and has several abandoned buildings and related structures.  The site has been divided into several areas to assist with easy identification of site features.  The Uplands Area includes the Ridge Top and Former Mine Areas, and the Lowlands Area includes the Plant Area, Southern Area, Northern Area and Railroad Spur Area.  Site features are identified on Figure 2.

Former processing facilities consisted primarily of two production buildings (“Mill Buildings”), baghouses adjacent to the two Mill Buildings (for air pollution control during operation), concrete bins used for unloading rail cars, an electrical building, an administration (office) building, a laboratory, a water supply plant, and a wastewater treatment plant (Figure 1).  The alloys produced in the Mill Buildings were made from chromium ores by smelting and refining in electric arc furnaces.  Four furnaces and two converters were housed in the Mill Buildings.  Piping used for transporting byproducts from the processing operations to the upland areas is present from the Mill Buildings to the upland areas north of the Mill Buildings.  Byproducts were either pumped as slurry through this piping or were hauled in trucks to the uplands.

Byproducts of the ferrochromium-alloy plant operations included slag and baghouse dust from the electric arc furnaces.  Baghouse dust from the electric arc furnaces is present beneath the baghouses and may be present in and around the Mill Buildings.  An estimated 800,000 cubic yards (1 M tons) of slag has been deposited across large tracts of both the upland and lowland areas of the Site.

The topography of the Site rises about 300 feet above the lowland floodplain of Cross Creek to a plateau surface.  The eastern portion of the Site consists of relatively flat lowlands comprising the main plant, while the western portion of the Site is an upland plateau formerly used for process material disposal, primarily slag, from chromium ore processing operations.  In the north, the Site is largely an upland plateau and consists of heavily wooded areas with slag from former Site operations.  The upland area also contains abandoned coal mine workings from the underground room-and-pillar Kolmont No.1 Coal Mine operated by the Wayne Coal Company in the 1930s.  The upland area includes a 5-acre area where chromite ore was stockpiled as part of the U.S. Government’s strategic mineral stockpile program.

## NPDES General Industrial Storm Water Permit

The Site’s NPDES General Industrial Storm Water Permit is effective until May 31, 2011.  A copy of the Ohio EPA Permit (No. OHR000004) is provided in Appendix A.  The Notice of Intent (NOI) and Acknowledgment letter from the OEPA are provided in Appendix B.

This SWPPP was prepared in accordance with the Ohio Environmental Protection Agency (Ohio EPA) General Permit Authorization to Discharge Storm Water Discharges Associated with Industrial Activity under the National Pollutant Discharge Elimination System (Effective Date June 1, 2006).  It was prepared using control measures and design guidance from Developing Your Storm Water Pollution Prevention Plan (USEPA, February 2009) and the Industrial Storm Water Fact Sheet Series (USEPA, 2006).

## Pollution Prevention Team

Names and contact information for those identified as members of the pollution prevention team are provided in the following table.

TABLE 1

Storm Water Pollution Prevention Team Members

| Owner: | Area of Responsibility |
| --- | --- |
| Cyprus Amax Minerals Company | Ms. Barbara Nielsen, will have overall responsibility for the work being conducted at the site and will ensure that all project and regulatory requirements are met. Ms. Nielson will be the official point of contact for all communications with OEPA, although she may authorize direct contact by other members of the project team as appropriate. |
| Cyprus Amax Project Manager (CAPM) |  |
| Barbara Nielsen |  |
| 333 North Central Avenue |  |
| Phoenix, AZ 84005 |  |
| (602) 355-8270 (o) |  |
| (480) 313-2895 (c) |  |
| SWPPP Coordinator | Area of Responsibility |
| Cyprus Amax Minerals Company | Mr. Suver is responsible for overall maintenance of the SWPPP and ensuring inspections and the Annual Comprehensive Site Compliance Evaluation are completed and documented. He is the primary regulatory point of contact. |
| Site Manager / SWPPP Coordinator |  |
| Soren J. Suver |  |
| 303 Walden Court Wexford, PA 15090-9455 |  |
| (724) 940-4368 (o) |  |
| (631) 374-8121 (c) |  |

## Regulatory and Emergency Contact Information

TABLE 2

Regulatory and Emergency Contacts

| Contact and Address |  | Name |  | Phone Number |  |
| --- | --- | --- | --- | --- | --- |
| Ohio EPA Southeast District Office |  | Aaron Wolfe |  | (740) 380-5277 |  |
| Ohio EPA Central Office Division of Surface Water Lazarus Government Center 50 West Town Street Columbus, OH 43215 |  | Jason Fyffe |  | (614) 728-1793 |  |
| Ohio Environmental Protection Agency |  | Site Project Manager |  | (740) 380-5251 |  |
| Ohio EPA Emergency Response |  | Duty Officer |  | 800-282-9378 614-224-0946 |  |
| Jefferson County Emergency Management Agency |  | Duty Officer |  | (740) 283-8600 (day) (740) 283-8600 (24-hr) |  |
| Fire or Medical Emergency |  |  |  | 911 |  |
| New Alexandria Fire Department |  | 3687 State Route 151 Mingo Junction, OH 43938 |  | (740) 598-3417 |  |
| ## Mingo Junction Fire Department |  | 501 Commercial St Mingo Junction, OH 43938 |  | (740) 535-1959 |  |
| Jefferson County Sheriff’s Department |  | Dispatcher |  | 911 740-283-8600 |  |
| Hillendale Fire Department |  | 2709 Wilson Avenue Steubinville, OH 43952 |  | 911 (740) 457-8460 |  |

# description of potential pollutant sources

## Drainage

A surface water drainage evaluation was conducted by Golder Associates Inc.  Information regarding surface water drainage in this SWPP was obtained from Golder’s Technical Memorandum entitled Satralloy Site Hydrologic Analysis Tech Memo, dated January 13, 2006, provided in Appendix C.  Surface water runoff in the Uplands Area is largely controlled by natural drainage features or through an existing storm sewer system that was constructed as part of the operational facilities and is present in the Plant Area.  A prominent ridgeline runs northeast-southwest across the site in the Uplands Area.  The natural slopes from the ridgeline range from 0.12 ft/ft to 0.50 ft/ft.

The Ridge Top Area consists of three sub-basins (Sub-basin A, Sub-basin B, and Sub-basin C).   These sub-basins flow from the ridgeline toward the northwest side of the site.  The anticipated direction of surface water flow follows natural slopes and topographic features to Cross Creek.

The Former Mine Area is composed of two sub-basins (Sub-basin D and Sub-basin E).  A large open bowl-like formation is present in the Former Mine Area in Sub-basin E.  Storm water drainage in this area flows down the hillside.  Sub-basin D begins in the Former Mine Area, flows through a natural crevice on the side of the hill to a storm water collection feature and discharges to Cross Creek.

The Southern Area is composed of three sub-basins (Sub-basin F, Sub-basin G, and Sub-basin H).   Each of these sub-basins originates at the top of a slope.  Storm water flows down the hillside following natural topography.  Sub-basins F and G flow to storm water collection features and discharge to Cross Creek.   Sub-basin H flows from the ridge top to the neighboring property.

In the Plant Area, storm water runoff generally flows from the area behind the buildings to natural crevices and directs flow to the existing storm sewer system that was constructed as part of the operational facilities.  This system consists of a series of channels, culverts and drop structures, and subsurface piping that convey storm water around the main plant area to Cross Creek.  Storm water that originates in the Plant Area generally flows southeast across the gravel covered areas, via diversionary structures and channels to Cross Creek.  A ditch along County Road 74 discharges into Cross Creek through several culverts under the road as well as a County installed drain tile.

The drainage patterns, flow directions and discharge locations for each sub-basin and the Plant Area are shown on Figure 3.  None of the drainage flow patterns are anticipated to have significant erosion associated with them.

## Inventory of Exposed Materials

No active industrial operations are occurring at the site.  Slag from former Site operations has been deposited in piles in the Former Mine and Ridge Top Areas.

The Mill Buildings and the electric building contain asbestos.  The Mill Buildings also contain lead materials.  Dust within the Mill Buildings and dust collected in piles behind the buildings under the former baghouses is impacted with chromium.  The slag piles contain varying concentrations of metals including chromium, arsenic and manganese.  These materials will be managed under the Construction SWPPP.

## Spills and Leaks

Given that the Site currently has no active industrial operations, spills or leaks associated with industrial activity are not anticipated at the Site.  No significant spills or leaks occurred at the Site in the 3 years prior to the date this SWPPP was prepared or amended.

## Sampling Data

No storm water data for the site is currently available.  Storm water sampling will be performed in accordance with Part V of the Industrial Storm Water Permit, as discussed in Section 5.

## Risk Identification and Summary of Potential Pollutant Sources

The slag piles are steep and in most cases devoid of vegetation.  The slag and baghouse dust are potential sources of particulates that may contain metals (chromium, arsenic and manganese) and/or elevated pH (due to lime residual in some slag) in storm water runoff.

# measures and controls

The control measures described in this section focus on the Ridge Top Area where natural surface water drainages flow into wetland areas along the western slope and in the Plant Area where storm water collection systems are in place.  The control measures, or Best Management Practices (BMPs), were selected considering a hydrologic analysis performed by Golder and guidance in the Rainwater and Land Development Manual (Ohio DNR, 1996).

## Erosion and Sediment Controls and Management of Runoff

The construction of new sediment ponds formed by building low earth dams across the drainages was evaluated.  However, the hydrologic analysis found that the ground surface is too steep to make earth dams practical (i.e., the volumes of the dams would be very large for an extremely small retention volume).  A review of historical aerial photographs and field observations indicate that the slag piles are cemented on the surface, erode very slowly, and materials generally have not traveled more than about 100 feet from the toe of the slag piles.  Consequently, given the small amount of sediment transport over the decades that the slag piles have been present and the low erosion potential of the slag, installation of erosion and sediment control measures is not considered necessary at this time.  Erosion and sediment patterns will be inspected as part of the annual Comprehensive Site Compliance Evaluation as discussed in Section 4.1.  Upon review of the results of the compliance evaluation, additional BMPs will be implemented, if necessary.

Runoff at the Site is managed through the natural drainage patterns and the existing subsurface storm water management features.  Natural features consisting of drainages in the hillsides collect and direct storm water flow toward Cross Creek.  The drainage patterns flow through heavily vegetated areas that in effect function as bioswales.

In the Plant Area, storm water runoff is also managed through the use of existing storm water features consisting of catch basins and subsurface piping and culverts (see Figure 3).   Silt fencing has been installed around catch basins, wetlands and other ground features that transport storm water to the catch basins to prevent transport of sediments.  These storm water features will be maintained.

## Good Housekeeping

Because no active industrial operations are occurring at the Site, housekeeping measures are not required.   Existing functional storm water features will be maintained.

## Preventative Maintenance

The erosion and sediment control measures described in Section 3.1 will be inspected in accordance with Section 3.5.  Controls measures or systems found to be damaged will be repaired.  If erosion and sediment control measures are found to be ineffective, new designs and measures will be evaluated and implemented as needed.

## Spill Prevention and Response

Given that the site is not in operation and no chemicals are used, spills and leaks are not a concern for this SWPPP.  However, contact information for individuals and agencies in the event of a spill or other emergency is provided in Section 1.4.

## Inspections

Inspections of the control measures will be performed by qualified individuals.  If present, storm water will be visually examined for the presence of suspended sediment, turbidity, discoloration, and oily sheen.  All existing control measures will be inspected to assess their condition and effectiveness and to identify any maintenance or repair actions needed to ensure the performance of their intended function.  If during inspections, other drainage channels are identified, control measures will be developed and implemented.

The inspector will evaluate and document the condition of the installed control measures.  A determination will be made regarding the necessity to modify or install additional control measures.

Any problems identified during the inspection will be corrected as follows:

- If the inspection reveals that a control measure is in need of repair or maintenance, it will be repaired or maintained within 30 days of the inspection.

- If the inspection reveals that a control measure fails to perform its intended function and that another, more appropriate control measure is required, the SWPPP will be amended and the new control measure will be installed within 12 weeks of the inspection.

- If it was identified that a control measure was not implemented as outlined in this SWPPP, the control measure will be implemented within 12 weeks of the inspection.  If it is determined that the control practice is not needed, the documentation will contain a statement of explanation.

### Site Inspection Frequency

Site inspections will be conducted at least once per quarter when activity is occurring at the Site.  During periods of no activity, only the annual Comprehensive Site Compliance Inspection will be conducted as discussed in Section 4.1.

### Site Inspection Documentation

The inspection form is provided in Appendix D of this SWPPP and describes the information collected during each inspection.  Completed inspection forms will be attached to the end of this SWPPP.

### SWPPP Activities Log

A site SWPPP log will be maintained for all on-site industrial SWPPP activities and will include:

- A record of the implementation of the SWPPP requirements.

- Site inspections.

- Control measure maintenance activities.

- Modifications to control measures or implementation of additional control measures.

A SWPPP Activities Log that describes the actions taken in response to inspections will be maintained, and is provided in Appendix D.

## Training

The Owner and Contractor are responsible for advising employees and subcontractors working on this project of the requirements outlined in this SWPPP prior to working on site and annually thereafter.  Particular emphasis should be placed on ensuring that employees and subcontractors do not damage control measures and do not introduce pollutants into the storm system or on-site waters.  All personnel will sign the Storm Water Pollution Prevention Training Log provided in Appendix E.  Training records will be maintained in the site trailer.

## Log of Changes to the SWPPP

If an inspection identifies that modifications to the SWPPP are necessary, the changes and the reasoning for the changes will be outlined on the SWPPP Amendment Log provided in Appendix F.

## Maintaining and Updating the SWPPP

A copy of this SWPPP will be kept in the Site trailer.  The SWPPP will be maintained, updated, and implemented in accordance with Part IV.C of the Industrial Storm Water Permit.

The SWPPP will be amended whenever there is a change in the design, construction, operation, or maintenance at the Site that has, or could have, a significant effect on the discharge of pollutants to offsite waters.

The SWPPP will be amended if during inspections or investigations conducted by the owner (or their authorized representative) or the Ohio EPA, it is determined that the SWPPP is ineffective in achieving the objectives of controlling pollutants in storm water discharges.  The SWPPP will be amended as necessary to include additional or modified control measures to correct the problems identified.  The amendments will be submitted to Ohio EPA if requested.

## Recordkeeping and Internal Reporting Procedures

### Records Retention

This SWPPP will be retained for the life of the permit.  Records of all monitoring information, copies of all reports required by the storm water permit, and records of all data used to complete the Notice of Intent shall be retained for a period of at least six years from the date of the measurement, report, or application.  Storm water monitoring data collected pursuant to the storm water permit will be retained for a period of six years from the date of sample collection or for the term of the storm water permit, whichever is greater.

### Access to Plans and Records

This SWPPP, a copy of the General Industrial Permit, Notice of Authorization letter, and monitoring data will be retained on site and will be made available upon request to the regulatory agencies.

### Non-Storm water Discharges

The storm water discharge locations at the Site have been evaluated.  Groundwater seeps exist on Site, and these seeps can mix with storm water.  Samples of the groundwater seeps have been analyzed and were found to contain chromium, as outlined in the “Scoping Study Report for the Former Satralloy Site,” dated May 29, 2008.

# Plan compliance

## Comprehensive Site Compliance Evaluation

A comprehensive site compliance evaluation will be performed by the SWPPP Coordinator or designated alternate at least once per year.  This evaluation will include the following:

- Visual inspection of storm water control measures to ensure that they are operating effectively.

- Visual inspection of drainage points and their associated control measures for evidence of, or the potential for, storm water impacts (e.g., sediment build up) and structural problems.

- Review of the site to identify potential drainage areas or new locations of potential sources that could impact storm water.

- Comprehensive inspections will be documented in a report that is signed by the SWPPP Coordinator and maintained in the on-site trailer.

- The SWPPP will be reviewed and updated as needed in response to findings of the comprehensive compliance evaluation.

The annual site compliance evaluation form is provided in Appendix G.  The form will be completed, signed, and retained as part of this SWPPP after each annual comprehensive compliance evaluation.

## Consistency with Other Plans

This Plan has been developed to be consistent with the Construction SWPPP entitled Storm Water Pollution Prevention Plan for the General Construction Permit Ohio EPA Permit No.: OCH000003 at the Former Satralloy Site, 4243 County Road 74, Mingo Junction, Jefferson County, Ohio 43928.

As activities at the site change, this document will be updated as needed to be consistent with any other plans required for the Site.

## Additional Requirements for “Section 313 Water Priority Chemicals”

- No water priority chemicals are present or being managed on site.

## Additional Requirements for Discharges to MS4s

- The Site does not discharge storm water associated with industrial activity to an NPDES permitted municipal separate storm sewer system.

# STORM WATER QUALITY MONITORING

## Sample Analyses

Sampling will be performed in accordance with the requirements outlined in Part V.B of the General Industrial Storm Water Permit.

Samples will be analyzed for the following parameters:

- Oil and grease

- 5-day Biochemical Oxygen Demand

- Chemical Oxygen Demand

- Total Suspended Solids

- pH

- Lead, Total

- Cadmium, Total

- Copper, Total

- Arsenic, Total

- Chromium, Total

In addition, the following information will be noted in a logbook:

- Duration of storm event

- Precipitation

- Duration between storm events (time between the storm event when sampling is being conducted and the last storm event that produced rainfall greater than 0.1 inches).

- Estimated volume of water in gallons.

## Storm Water Drainage and Sample Locations

### Drainage Areas

The Site has a total of nine storm water drainage areas.  Eight of these areas drain storm water from the Site’s Uplands.  The ninth drainage area consists of the Plant Area, which is located in the Site’s Lowlands, and includes the former mill buildings, ancilliary buildings, and parking areas.  The storm water drainage areas are labeled A through I on Figure 3.

Three drainage areas, Drainage Areas A, B and C, drain the north side of the Ridge Top Area.  Drainage Area A flows toward Cross Creek, then curves at a ridge then flows in a southwestern direction and disperses on land.  Drainage Areas B and C discharge into Cross Creek.

Storm water flows from Drainage Area D into the existing storm water collection system in the Plant Area, Drainage Area I, which is located just north of the North Mill Building.

Drainage Areas F and G flow to the existing storm water collection system located behind the South Mill Building, which is also located in Drainage Area I.

Drainage Areas E and H are located such that storm water flow does do not discharge to Cross Creek.

### Discharge Locations

The Site has a total of 10 active locations where storm water flows from the drainage areas listed above and discharges near or directly into Cross Creek.  Table H-1, Storm Water Discharge and Sampling Location Information, provides a description of each of the storm water discharge location.  Table H-1 is located in Appendix H, and discharge locations are identified on Figure 3.

### Sample Locations

Initially, samples will be collected from each of the active storm water discharge locations and the samples will be analyzed for the parameters outlined in Section 5.1.  The analytical results will then be reviewed.

During subsequent sampling events, only one representative sample will be collected from each drainage area that discharges to Cross Creek.  Where several sample locations are representative of a certain drainage area, the sample(s) with the highest concentrations will be selected for future sampling.  The remainder of the samples will be eliminated from future sampling events.  For example, discharge locations GC1, GC2, GC3, and GC4 are representative of storm water discharging from the slag on the northeast portion of Drainage Area I.  It is anticipated that the analytical results for these samples will be similar.  Upon review of the data, the sample with the highest concentration will be selected as representative of these four discharge locations.

In addition, data will be reviewed to determine the effectiveness of the implemented control measures.   Records, including the reasoning for selecting samples for subsequent sampling events, will be maintained in accordance with Section 3.9.1 of this plan.

APPENDIX A

NPDES Permit

APPENDIX B

Appendix B	NOI and Ohio EPA Acknowledgment Letter

APPENDIX C

Satralloy Site Hydrologic Analysis Tech Memo

APPENDIX D

Inspection Form and Follow-up activities log

Former Satralloy Site

Industrial SWPPP Inspection Form

Instructions:  This inspection form will be used in conjunction with the Former Satralloy Site SWPPP Control Measure checklist.  The checklist will identify the location of each control measure.  The SWPPP Inspection form will be used to log data associated with each inspection.

Date: ________________________   Time: ______________

Inspector’s Name: __________________________________ Title: _____________________________________

Inspector’s Qualifications:  ________________________________________________________________________________________________________________________________________________________

Weather information:  _________________________________________________________________________

In my judgment, the site is (select one) IN / OUT of compliance with the site’s SWPPP.

Recommended Actions:

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

“I certify that this report is true, accurate, and complete to the best of my knowledge and belief.”

Inspector’s signature:  _________________________________________________

Former Satralloy Site

Industrial SWPPP Checklist

Control Measure Reviewed	Acceptable	Requires	Comments

Maintenance

O  Discharge Location SC1		O_______	O_______	_______________________

O  Discharge Location SC2		O_______	O_______	_______________________

O  Discharge Location GC1		O_______	O_______	_______________________

O  Discharge Location GC2		O_______	O_______	_______________________

O  Discharge Location GC3		O_______	O_______	_______________________

O  Discharge Location GC4		O_______	O_______	_______________________

O  Discharge Location GC5		O_______	O_______	_______________________

O  Discharge Location GC6		O_______	O_______	_______________________

O  Discharge Location GC7		O_______	O_______	_______________________

O  Discharge Location GC8		O_______	O_______	_______________________

O  Discharge Location GC9		O_______	O_______	_______________________

O  Discharge Location GC10		O_______	O_______	_______________________

O  Site Ditches			O_______	O_______	_______________________

O  Catch Basins			O_______	O_______	_______________________

O  Silt Fencing			O_______	O_______	_______________________

Condition of Slag Movement:

O  Drainage Area A			O_______	O_______	_______________________

O  Drainage Area B			O_______	O_______	_______________________

O  Drainage Area C			O_______	O_______	_______________________

O  Drainage Area D 			O_______	O_______	_______________________

O  Drainage Area E 			O_______	O_______	_______________________

O  Drainage Area F 			O_______	O_______	_______________________

O  Drainage Area G			O_______	O_______	_______________________

O  Drainage Area H			O_______	O_______	_______________________

O  Drainage Area I			O_______	O_______	_______________________

Follow-up Activities: For each control measure above, list follow-up activities on the Follow-up Activities Log.

Project Name:	  Former Satralloy Site

SWPPP Contact:    Soren Suver

| Inspection Date | Inspector Name(s) | Description of Control Measure Deficiency | Follow-up Activities Needed (including planned date/responsible person) | Date Activities Completed/ Responsible person |
| --- | --- | --- | --- | --- |

APPENDIX E

Training Log

Storm Water Pollution Prevention Training Log

Project Name:

Project Location:

Instructor’s Name(s):

Instructor’s Title(s):

Course Location: 	  Date:

Course Length (hours):

Storm water Training Topic:  (check as appropriate)

|  | Erosion and Sediment Controls |  |  |
| --- | --- | --- | --- |
|  | Emergency Procedures |  |  |
|  | Good Housekeeping |  |  |

Specific Training Objective:

Attendee Roster:  (attach additional pages as necessary)

| No. | Name of Attendee | Company |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 12 |  |  |
| 13 |  |  |
| 15 |  |  |
| 16 |  |  |
| 17 |  |  |
| 18 |  |  |

APPENDIX

F

SWPPP AMENDMENT log

| Amendment No. | Description of the Amendment | Date of Amendment | Amendment Prepared by (Name(s) and Title) |
| --- | --- | --- | --- |

APPENDIX G

annual site compliance evaluation form

Annual Storm Water Compliance Evaluation Form

Facility Name: Inspection Date:

Location: Inspector’s Name:

“I certify that this report is true, accurate, and complete to the best of my knowledge and belief.”

Inspector’s Signature:

Has there been any new construction that would affect the site map or drainage conditions at the facility? Yes No N/A

Are there any changes in facility operations that could be identified as new source areas for contamination of storm water? Yes No N/A

Visually inspect drainage points for evidence of, or the potential for, storm water contamination (e.g., sheen on the surface water, discoloration, odors, etc.) and structural problems. Is there evidence of or the potential for storm water contamination? Yes No N/A

Review site for new locations of potential storm water contamination sources. Yes No N/A

Are Best Management Practices implemented to reduce or eliminate contamination of storm water from source areas at the facility? Yes No N/A

Are the current storm water management measures, sediment and erosion control measures, and other pollution prevention control measures identified in the SWPPP implemented effectively and adequately maintained? Yes No N/A

Are there any materials at the facility that are handled, stored, or disposed in a manner to allow exposure to storm water that are not currently addressed in your SWPPP? Yes No N/A

Are particulates from the slag piles or other areas being adequately controlled by current control measures? Yes No N/A

Are regular monthly inspections being completed? Yes No N/A

Do you see spots, pools, puddles, or other traces of oils, grease, or other chemicals on the ground? Yes No N/A

Are there any maintenance or material handling activities conducted outdoors that have not been addressed in your SWPPP? Yes No N/A

Do you see leaking equipment, pipes or containers? Yes No N/A

Do drips, spills, or leaks occur when materials are being transferred from one source to another? Yes No N/A

Are drips or leaks from equipment or machinery being controlled? Yes No N/A

Are absorbent materials (floor dry, kitty litter, etc.) regularly used in certain areas to absorb spills? Yes No N/A

Are significant changes needed for the SWPPP to correct plan inadequacies to effectively control a discharge of contaminated storm water? Yes No N/A

Have any new “Non-Storm Water” Discharges been introduced or been identified? Yes No N/A

The SWPPP will be reviewed and updated as needed in response to findings of the compliance evaluation.

Comprehensive inspections must be documented in a report that is signed and maintained in the storm water file.

APPENDIX H

Storm water discharge areas and sampLing locations