Introduction

The following procedures describe the general methodologies that will be used in the field to collect freshwater fish.

Pre-Collection

Staff assigned the responsibility of collecting fish will be provided with the following information:

Work documents

Water body name and site maps

Number and size of each species to be collected

Collecting and processing procedures

Special instructions (if any)

Appropriate fisheries office contact

Sampling permits and licenses

Equipment

The following collection equipment and materials will be available, as required, during fish sampling:

Personal protective equipment

Boat, engine, life jackets, anchors, buoys, and rigging

Electrofishing equipment

Dip nets with non-conductive handles

Chest or hip waders

Live well, cooler, or 5-gallon bucket

Measuring board or ruler

Top-loading electronic and suspended-weight spring balances

Insulated coolers with ice

Plastic sealable bags and indelible ink markers

Camera

Global positioning system (GPS)

Field notebook

Field Notes

Field notes will be recorded during sampling activities, and at a minimum, will include the following:

Names of field crew and oversight personnel

General weather conditions

Date, time, and general capture location (GPS if specified)

Capture technique

Sample duration

General observations of fish habitat, abundance, and diversity

Photograph number when pictures are taken (if necessary)

Collection Procedures

Fish will be collected using trained personnel and approved sampling techniques.  State personnel (conservation officers) will be notified of the sampling activities and any required permits will be obtained prior to going into the field.  Only those target species identified in the work plan and scientific collectors permit will be retained.  Collection of other species may occur when target species are absent, and as approved.  Non-target species will be released back to the system.  Fish will be collected using electrofishing techniques.  The following procedures will be used, as necessary, to collect fish:

Electrofishing

This section describes the procedures associated with the use of boat-mounted and barge-mounted electrofishing units.  Electrofishing activities will be temporarily halted when any persons, pets, or livestock are observed in the water or on the shore in close proximity to the electrofishing unit.

Boat-Mounted Electrofishing

The following procedures describe the use of an electrofishing boat to collect fish:

The field crew will don personal protective equipment (life jackets, non-conductive shoes and gloves, etc.), set up the electrofishing equipment, and test it upon arrival at the site.

The boat operator will be responsible for control of the boat, and operation of the control equipment and generator.  The remaining field crew will operate from the front of the boat and will be responsible for control of the on-off floorboard switch, and capturing the fish.

Electricity will be applied to the water by actively maintaining the on-off switch in the closed position while the generator and control equipment are operative.

The electrical current will be set to stun the fish, but should not cause mortality.

Target species of appropriate size will be collected using non-conductive dip-nets and will be placed in a live-well with fresh water until they can be transferred to a cooler with ice.

Barge-Mounted Electrofishing

The following procedures describe the use of a barge-mounted electrofishing unit to collect fish:

- The field crew will don personal protective equipment (life jackets, non-conductive waders and gloves, etc.), set up the electrofishing equipment, and test it upon arrival at the site.

The barge operator will be responsible for pushing the barge (6-foot in length), and operation of the control equipment and generator.  The remaining field crew will walk in front of the barge with anodes and will be responsible for control of the on-off switch on the anode handles, and capturing the fish.

Electricity will be applied to the water by actively maintaining the on-off switch in the closed position while the generator and control equipment are operative.

The electrical current will be set to stun the fish, but should not cause mortality.

Target species of appropriate size will be collected using non-conductive dip-nets and will be placed in a cooler with fresh water until they can be transferred to a cooler with ice.

Sample Handling

The following identifies the temporary storage procedures that will be used to preserve fish in the field prior to sample processing, handling, and shipment to the laboratory:

- Measure or weigh each fish after collection, as necessary, to ensure that appropriate sized fish are taken and that minimum sample mass requirements are satisfied.

- Count the number of fish to ensure that the correct amount is taken.

- Transfer fish to sealable plastic bags (if not done previously) and label with sampling date and capture location, and place in coolers with ice until field processing can occur.  Large fish that do not fit into plastic bags may be placed on ice in clean coolers that are clearly labeled.

Golder Associates Inc.

18300 NE Union Hill Road, Suite 200

Redmond, WA  98052 USA

Tel:  (425) 883-0777

Fax:  (425) 882-5498