Introduction

The following procedures describe the general methodologies that will be used in the field to process (handle, pack, and ship) whole-body fish samples for laboratory analysis.

Equipment

The following equipment and materials will be available, as required, to process whole-body fish samples:

Measuring board or ruler

Top-loading electronic and suspended-weight spring balances

Forceps or knife

Storage envelopes and glass vials

Sealable plastic bags and freezer paper

Tape (duct, strapping, masking/freezer, and clear packing)

Indelible ink markers

Cleaning and decontamination materials

Potable water

Insulated coolers with ice

Forms (chain-of-custody, custody seal, address label, and air-bill)

Camera

Field notebook

Field Notes

Field notes will be recorded during processing of fish samples, and at a minimum, will include the following:

Names of field processing crew and oversight personnel

Date and time of processing

Sample numbers that correspond to analysis, sampling date, and collection location

Body lengths and weights, and number of organisms per sample

Gross external abnormalities and visible parasites

The collection of bony structures for aging (where applicable)

Photograph number when pictures are taken (if necessary)

Handling, Packing, and Shipping

The following identifies the procedures that will be used to handle, pack, and ship whole-body fish samples:

Handling

All samples will be given a sample identification number that will be recorded in the field notebook, and that corresponds to the sample analysis, sampling date, and collection location.

Chain-of-custody forms, custody seals, address labels, and air-bill forms will be initiated.  Chain-of-custody forms will identify the tissue sample preparation procedure and chemical analysis that the lab will follow.  A copy of the completed chain-of-custody form and air-bill form will be retained by the sampler.

To begin processing, sediments, soil, and other debris will be removed from the fish samples by hand-picking or by rinsing with potable water.

Each fish in a composite sample will be measured to the nearest millimeter and weighed to the nearest gram.  Total length will be measured for each fish as opposed to standard length or fork length.

Whole-body composite fish samples will have sufficient sample mass to meet the minimum sample mass requirements for chemical analysis.

Photographs will be taken, as required, and any external abnormalities or visible parasites will be noted in the field notebook.

Hard body parts will be removed from fish in the field, as required, as long as it does not compromise the integrity of the sample.  If integrity is compromised, the laboratory may instead be asked to remove the bony structure.  The structure will be archived for possible aging at a later date.

Each fish sample will be wrapped in a sealable plastic bag and then in freezer paper to maintain sample integrity.  The freezer paper will be secured with masking or freezer tape.  The sample identification number will be written on both the plastic bag and freezer paper with a waterproof marker.

Samples will be separated by organism type and by sample location, and will be placed into large sealable plastic bags in preparation for packing.

All equipment will be cleaned with a laboratory grade detergent and a potable water rinse between samples, species groups, and/or between locations.

Packing

- Coolers used for transport will be duct-taped at the drain plug on the outside and inside of the cooler.

- Wet ice will be packaged in sealable plastic bags (double-bagged with the outer seal duct-taped) and placed in the bottom of the cooler.  The sealed fish samples will be placed inside the cooler with enough room for additional ice bags to be placed on top.

- The completed chain-of-custody form will be placed into a plastic bag and duct-taped to the inside of the cooler lid.

- The cooler will be closed and fastened with duct tape around the seam of the lid to prevent water leakage and with strapping tape around the entire cooler to prevent it from opening during transport.

- A completed custody seal will be placed across the seam of the cooler lid.  A completed address label will be placed on top of the cooler.  Both will be taped-over using clear packing tape.

Shipping

- Samples with holding time requirements will be shipped to the laboratory by hand or by express carrier within 48 hours or less from the date of sample collection.  Samples that are fixed with preservative, or that do not have stringent holding times, will be delivered in a timely manner.

- The laboratory will be notified of the shipment and will be contacted immediately following the arrival date to ensure that delivery has occurred.

Golder Associates Inc.

18300 NE Union Hill Road, Suite 200

Redmond, WA  98052 USA

Tel:  (425) 883-0777

Fax:  (425) 882-5498