appendix H
field sampling procedures

May 8, 2013	Project No. 053-1695.140

Table of Contents

1.0	Introduction	1

2.0	Documentation	2

2.1	Field Notebooks	2

2.2	Field Meters	2

2.3	Photographic Documentation	3

3.0	Groundwater Sampling Procedures	4

3.1	Groundwater Monitoring Well Installation	4

3.2	Groundwater Monitoring Well Development	4

3.3	Groundwater Monitoring Well Sampling	5

3.3.1	Low-Flow Purging and Sampling	6

3.3.2	Minimum/No Purge Sampling	7

4.0	Surface Water Sampling Procedures	10

4.1	Site Seep and Surface Water	10

4.1.1	Site Seeps	10

4.1.2	Site Surface Water	11

4.2	Cross Creek Surface Water	11

5.0	Creek Sediment Sampling Procedures	12

6.0	Sampling Procedures for Soil and Other Solids	13

6.1	Near-Surface Samples	13

6.2	Soil from Borings	13

7.0	Decontamination and Investigation-Derived WasteS	15

7.1	Drill Rig and Drilling Equipment	15

7.2	Water Quality Meters and Water Level Meters	16

7.3	Groundwater Sampling Pumps	16

7.4	Other Sampling Equipment	17

7.5	Investigation-Derived Waste	17

7.5.1	Borehole Cuttings	17

7.5.2	Water	17

7.5.3	Personal Protective Equipment and Disposable Sampling Equipment	18

8.0	References	19

List of Attachments

Attachment H1	Fish Sampling Procedures

Attachment H2	Field Processing and Handling Procedures for Fish Samples

Introduction

This document provides the field sampling procedures for the Former Satralloy Site (Site) located in Cross Creek Township, Jefferson County, Ohio.  A description of the Site and the project background can be found in the Introduction (Section 1) of the Remedial Investigation/Feasibility Study (RI/FS) Workplan.

The purpose of these procedures is to identify and describe sampling procedures that will be performed during the RI.  These procedures will be used in conjunction with the Quality Assurance Project Plan (QAPP) for the RI at the Former Satralloy Site (Golder 2012).  Quality control samples will be used to monitor sampling and laboratory performance and will include duplicate blanks per the QAPP.  The sample containers for both the primary and quality control (QC) samples will be appropriately preserved in accordance with the QAPP and shipped to the analytical laboratories following chain-of-custody (COC) procedures.

Attachments H1 and H2 provide procedures related to fish sampling.

Documentation

This section provides the procedures and protocols proposed to be used to document pertinent field information and data.

Field Notebooks

Information pertinent to the field investigation will be recorded in bound and numbered field notebooks and/or field information forms.  Field notes should be legible.  Known errors should be crossed out with a single line and initialed.  Each sampling team will be assigned a notebook.  Field records should include the following information:

Date

Project or site name

Time of each data entry

Description of work being performed that day

Names and affiliations of personnel at location

Weather conditions on site

Location and type of activity

Visual observations

Pertinent field data (and any other measurements)

Serial numbers, if any, on seals, transportation cases, and equipment

Photographs taken, including date, time, direction faced, description of subject or activity, and sequential number of the photo will be recorded in the field notebook or on a photo-log sheet

Specific sample information (e.g. sample type, sample depth) will be documented in the field notebook and/or on Sample Integrity Data sheets and on the chain of custody form (COC).  All field notebooks will be standard engineering-type hardbound books.  The contents of the field notebooks will be photocopied on a weekly basis so that copies of field notes can be kept in appropriate project files.

Field Meters

Calibration of field meters used during the project will be documented, including:

Analyst name

Date and time of calibration

Date and time and results of calibration checks

Instrument type, model number, and serial number (if present)

Manufacturer, concentration, and lot number of calibration standards which are used

A multi-parameter meter with a flow cell, such as the Horiba® U-22 instrument, will be used to for measurement of field parameters during groundwater monitoring: pH, dissolved oxygen, conductivity, temperature, oxidation reduction potential (eH) and turbidity.  Water level measurements will be determined with water level meters.  The field meters will be calibrated in accordance with the manufactures’ specifications.

Photographic Documentation

Prior to use, the camera will be checked to ensure the correct date and time are recorded for each photograph (if the camera has a date and time feature).  When photographs (digital or film) are taken, field personnel will record time, date, site location, general direction faced, sequential number of photograph and roll number (if pertinent), and a brief description of the subject in the field notebook or on a photo-log sheet.  This information will be included with the film photographs and/or printed digital photographs and stored in the project files.

Groundwater Sampling Procedures

Groundwater Monitoring Well Installation

Monitoring wells will be installed in general accordance with “Chapter 7:  Monitoring Well Design and Installation” (OEPA 1995) of the Ohio Environmental Protection Agency (OEPA) Technical Guidance Manual.  The installation will be performed by a driller licensed by the State of Ohio, and the work will be monitored by a geologist or engineer.  After installation, all wells will be surveyed by a surveyor licensed in the State of Ohio.

At each soil boring location, hollow stem auger (HSA) drilling methods will be used to advance the borehole to the required depth.  Continuous sampling will be used during drilling.  Samples for analysis will be selected based on visual indications as specified in the Field Sampling Plan.  Boreholes in rock will be drilled using air rotary or similar methods (where core is not required) and wire-line diamond coring or sonic coring.  Well screen intervals will be determined on field observations during drilling.

Following drilling, all wells will be completed using 2-inch internal diameter (ID), Schedule 40 polyvinyl chloride (PVC) casing and screen.  Well screen lengths will not exceed 10 feet.  The implementation of the well construction will be determined in the field by the supervising geologist/engineer, based on the stratigraphy and field screening results.  The OEPA will be contacted prior to implementing any significant change to well construction.  The well screen slot size will be 0.010 inch (No. 10 slot) and well casings will extend from the top of the well screen to approximately 2 feet to 3 feet above the ground surface.  A sand pack, comprised of clean quartz sand, will be placed in the bottom of the annulus around the well screen and will extend 2 to 3 feet above the well screen.  The sand pack material will be such that 90 percent by weight is larger than the screen slot size of 0.010 inch.  One foot of filter pack (i.e., fine-grained sand pack) will be placed and extend above the sand pack to reduce likelihood that grout will mix with the sand pack.  At least 3 feet of bentonite will be placed above the sand pack to provide a seal between the sand pack and grout.  A cement/bentonite grout (6 percent to 10 percent bentonite) will then be placed using a tremie pipe (side discharge) from above the filter pack to the ground surface.  A protective steel casing with a locking cap will be at least 3 feet below ground surface (bgs) and extended approximately 30 inches above ground surface.  Each new well will include a weep hole drilled through the protective casing near ground level.

Groundwater Monitoring Well Development

After installation, each well will be developed using a pump or other purging method in general accordance with “Chapter 8:  Monitoring Well Development” (OEPA 2004) of the OEPA Technical Guidance Manual.  Well development will not begin until the well seal/grout sufficiently cures and settles (generally 24 to 48 hours after placement).  Well development will include cycles of surging and purging and will continue until measured water quality parameters of the discharge water stabilize.  To determine stability, the field parameters of pH, specific electric conductance (conductivity), temperature, and turbidity will be monitored with a field meter at least every 5 minutes.  Stabilization will be considered complete when over three consecutive readings are recorded within the following ranges:

pH is within +/- 0.1 Standard Units (SU)

Conductivity is within 3 percent

Temperature is within +/- 1 degree Celsius (oC)

Turbidity is within 10 percent, or is less than 10 nephelometric turbidity units (NTU)

Field parameter measurements will be made with a flow-through cell prior to the purge water being exposed to the atmosphere.  The results will be recorded on field information forms.

The depth to the bottom of the well will also be measured.  In order to minimize potential cross-contamination and disturbance to sediments, which may have accumulated in the bottom of the well, well depth measurements will only be taken after well development.

Groundwater Monitoring Well Sampling

Newly installed monitoring wells will not be sampled any sooner than seven days after the well has been developed.

Equipment

Groundwater sampling equipment will be constructed of inert materials such as stainless steel or Teflon.  Proper equipment decontamination procedures, as discussed in Section 3, will be followed to minimize the potential for cross-contamination.  The following equipment may be needed:

Grundfos® Redi-Flo2 submersible pump (or equivalent)

Redi-Flo Variable Frequency Drive pump controller (or equivalent)

Portable electrical generator

Water quality meter (Horiba® U-22 instrument or equivalent) capable of measuring pH, conductivity, temperature, turbidity, ORP, and DO

Field meter flow cell

Water level meter

Teflon-lined polyethylene tubing

Latex and/or nitrile gloves

5-gallon buckets and lids or other suitable containers (to temporarily containerize water)

Groundwater field information forms

COC forms

Field notebooks

Well construction information

Sample containers and preservatives

Cooler with ice

Sample location map

Well keys

Low-Flow Purging and Sampling

In general, groundwater sampling will be conducted at the Site using low-flow purging and sampling techniques in general accordance with “Chapter 10:  Ground Water Sampling” (OEPA 2006) of the OEPA Technical Guidance Manual.  This procedure is summarized below.

Prior to purging, a water level meter will be used to measure the depth from top of the inner well casing at the survey point to the top of water surface.  Measurements shall be made to the nearest 0.01 feet.  The water level meter will be cleaned prior to each measurement in accordance with the equipment decontamination procedures included in Section 3.2.

The monitoring wells will be purged and sampled with a stainless steel Grundfos® Redi-Flo2 submersible pump (or equivalent).  The low flow purge and sampling procedures are described below.

The submersible pump will be carefully lowered into the well to the approximate midpoint of the well screen or open interval.  Separate dedicated Teflon-lined polyethylene tubing will be used for each well.  In instances where the screened or open interval is not completely saturated, the pump will be lowered to the midpoint of the saturated interval where possible.  Care will be taken to collect representative samples by placing the pump adjacent to a water-bearing zone.

Each well will be initially purged at a rate of 200 to 500 milliliters per minute (mL/min).  The water level in the well will be monitored during pumping, and ideally the pumping rate should equal the well recharge rate with minimal water level drawdown in the well.  The pumping will only be increased as long as there is no drawdown (not to exceed 1,000 mL/min).  Best efforts should be made in order to minimize well drawdown by adjusting the flow when necessary and frequently monitoring the water level during purging.  Care will be taken to ensure that at least 1 foot of water is maintained over the pump intake so there is no risk of the pump suction being broken, or entrainment of air in the sample.  The pumping rate adjustments and depth(s) to water readings will be recorded on the field information form.

During purging, the water level and field parameters (pH, conductivity, temperature, and turbidity) will be monitored with a water quality meter approximately every 5 minutes until the parameters have stabilized.  Stabilization is considered achieved if pH is within +/- 0.1 SU, conductivity is within 3 percent, temperature is within +/- 0.5 oC, turbidity is within 10 percent (or is less than 5 NTU) over three consecutive readings.  Field parameter measurements will be made with a flow-through cell prior to the purge water being exposed to the atmosphere.  The results will be recorded on field information forms.

If drawdown does not stabilize, the flow rate may be reduced to 100 mL/min, however this reduced rate may result in heating of the purge water.  If this occurs, the rate will be increased to a point where temperature is stable (+/- 0.5 °C).  If drawdown exceeds 2 feet and still does not stabilize, low-flow purging shall be stopped.  The well will be sampled using the minimum/no purge sampling method described in the section below.

Once the field parameters have stabilized, final measurements will be recorded.  The flow cell will be disconnected, purging will continue for approximately 2 minutes, and then the sample will be collected directly from the end of the pump discharge tubing.  All sample bottles will be filled by allowing the pump discharge to flow gently down the inside of the bottle with minimal turbulence.  Each bottle will be capped as it is filled.

Immediately after sample collection, sample bottles will be placed on ice and maintained at approximately 4 (+/-2) °C.

The following information will be recorded on a groundwater field information form for each monitoring well sampled:

Before Purging

Date

Well ID

Depth to water

Purging

Start and end time for purging

Purge method

Purge rate

Depth to water

pH, conductivity, temperature, turbidity, ORP, and DO (approximately every 5 minutes)

Sampling

Start time for sampling

Weather conditions

Sampling device

Sampling method

Pertinent observations regarding sample characteristics (turbidity, color, etc.)

Minimum/No Purge Sampling

If a well cannot be sampled using low-flow purging due to excessive drawdown (greater than 2 feet), in accordance with Chapter 10 of the OEPA Technical Guidance Manual, or if based on previous sampling data, a well is known to produce at less than 100 mL/min, then the following procedure for minimum/no purge sampling shall be used.

Prior to starting this procedure, two calculations are required:  sampling system volume and limit of acceptable drawdown.

To calculate the sampling system volume, the submersed tubing length must be determined (subtract the depth-to-water reading from the depth to the top of the pump).  The tubing volume will be calculated by multiplying the submersed tubing length by one of the following tubing diameter volume factors:

For 1/4” inner diameter (ID), use 10 milliliters per foot (mL/ft)

For 3/8” ID, use 22 mL/ft

For 1/2” ID, use 39 mL/ft

An additional 50 mL should be added to account for the estimated volume of water in the pump.

To avoid drawing stagnant water from the column above the well screen (or open interval), the amount of drawdown will be monitored during minimum/no purge sampling.  The limit of acceptable drawdown will be used to determine when sampling shall cease.  To determine the limit of acceptable drawdown, the height of the screened water column above the pump must be calculated.  First, the depth to the top of the screened interval will be determined from the well construction.  Next, the depth to the top of the screen will be subtracted from the depth to the pump to determine the length of screen above the pump.  Finally, an additional 2 feet will be subtracted to act as a safety margin.  This result will be the allowable drawdown in the well during sampling.  These calculations will be documented on the field information form.

The following procedure will be followed for the collection of a minimum/no purge sample:

- The submersible pump, with dedicated Teflon-lined polyethylene tubing, will be carefully lowered into the well and positioned approximately 1 to 2 feet from the bottom of the well.  Care will be taken to avoid contacting the bottom of the well with the pump.  This contact may suspend sediments which have previously settled to the bottom of the well.  If the pump does contact the bottom of the well, the sediments will be allowed to resettle for a minimum of 1 hour prior to sampling.  Please note that if the pump was already placed at the mid-point of the screen for low-flow sampling, the pump should not be lowered prior to sampling, unless sampling is halted to allow the water level to recharge.

Each well will be initially purged at a rate of 200 mL/min.  The calculated sampling system volume will be purged into a transfer vessel for proper disposal.  After this volume is purged, sampling will commence.

The samples will be collected directly from the end of the pump discharge tubing.  All sample bottles will be filled by allowing the pump discharge to flow gently down the inside of the bottle with minimal turbulence.  Each bottle will be capped as it is filled.

The water level in the well will be monitored during sampling.  If the water level drops to the pre-determined limit of acceptable drawdown, sampling will be halted, even if all parameters have not been collected.  Once the well recharges sufficiently, sampling may continue.  Sampling may occur over several days.  At a minimum, sampling should be attempted every 24 hours until completed.

After all analytical samples are collected, a single set of field parameters (pH, conductivity, temperature, turbidity, ORP, and DO) will be measured using the flow-through cell and the water quality meter and recorded on field information forms.  Field parameters may not be measured if there is insufficient volume.

If the well cannot be sampled due to water level drops below the pre-determined limit by the above procedure, in accordance with the OEPA Technical Guidance Manual, the well will be purged until it is dry.  Time will be allowed for the well to recover and sampling will occur within 24 hours of initial purging. After all analytical samples are collected as described above (Step 3), a single set of field parameters will be determined as detailed above (Step 5).

Surface Water Sampling Procedures

Surface water samples will be collected from Site seeps (i.e. springs), Site surface water bodies (e.g. ponded water, streams, etc.), and Cross Creek.  Surface water sampling equipment will be constructed of inert materials.  Proper equipment decontamination procedures will be followed to minimize the potential for cross-contamination.  Quality control samples and sampling shipping and preservation will be in accordance with the QAPP.

Equipment

Water quality meter (Horiba® U-22 instrument or equivalent) capable of measuring pH, conductivity, temperature, turbidity, oxidation-reduction potential (ORP), and dissolved oxygen (DO)

Field meter probe sleeve

Filter vessel

Compressed nitrogen cylinder and regulator

Disposable 0.45 micron filters

Waders

Shovel

Latex and/or nitrile gloves

COC forms

Field notebooks

Sample containers and preservatives

Cooler with ice

Sample location map

Site Seep and Surface Water

Site Seeps

Site seeps are considered representative of groundwater.  Groundwater at seeps will be sampled by use of buried passive sampling methods by this procedure.  Samplers will be inserted into capped PVC cylinders and lowered into small trenches excavated by hand below the water table.  The PVC cylinder will be perforated to ensure proper flow of water and good connection between the sampler and groundwater.  A durable plate will be placed above the PVC cylinder to separate surface water flow from groundwater in equilibrium with the sampler.

Samples will be collected with regenerated-cellulose dialysis membrane samplers or rigid, porous polyethylene (RPP) samplers (such as manufactured by Columbia Analytical Services).  After an appropriate deployment length to ensure concentrations inside the sampler to equilibrate with groundwater constituents (minimum of 7 and 14 days, respectively, for hexavalent chromium in dialysis sampler and RPP sampler), the samplers will be retrieved.  Samples collected from the RPP samplers will be filtered through a 0.45 micron filter directly into a clean sample container provided by the lab.  Samples collected from dialysis samplers do not require filtration due to the 0.0018 micron pore size and will be sampled directly into clean sample containers provided by the lab.

Site Surface Water

Grab samples from Site surface water (other than seeps) will be collected as described in this section.  Each sampling location will be inspected prior to collection.  In the case of water too shallow to facilitate the filling of sample bottles, a sump will be dug with a shovel at the sampling location.  Each sample will be collected by directly dipping the sampling container into the surface water (either ponded or flowing).  To collect the field-filtered samples, a decontaminated filter vessel will be used with compressed nitrogen and disposable 0.45 micron filters.  Alternatively, field-filtered samples will be collected as follows:

Fill a clean sample container with unfiltered sample water

Use a peristaltic pump equipped with a short piece of tubing and an in-line 0.45 micron filter to pump the water through the filter and directly into a clean sample container provided by the lab

After collecting the samples, a water quality meter will be used to collect field parameter readings (i.e. pH, conductivity, temperature, turbidity, ORP, and DO).

Cross Creek Surface Water

The Cross Creek surface water samples will be collected by directly dipping the sampling containers into the surface water flow of Cross Creek.  To eliminate potential impact from disturbed sediments, the furthest downstream sampling location will be collected first.  Subsequent samples will be collected in an upstream direction with the most upstream sample location collected last.  At each location, the sampler may need to enter the creek with waders to facilitate the collection of the samples.  He or she should approach the sampling location from downstream, collecting the samples upstream of his or her body.  To collect the field-filtered samples, a decontaminated filter vessel will be used with compressed nitrogen and disposable 0.45 micron filters.  Alternatively, the peristaltic pump and in-line 0.45 micron filter method described in Section 4.1 may be used.  After collecting the samples, a water quality meter will be used to collect field parameter readings (i.e. pH, conductivity, temperature, turbidity, ORP, and DO).

Creek Sediment Sampling Procedures

The following Section presents procedures for collection sediment samples from Cross Creek in general accordance with the Sediment and Sampling Guide and Methodologies (OEPA 2001).  Sediment sampling equipment will be constructed of inert materials.  Proper equipment decontamination procedures will be followed to minimize the potential for cross-contamination.  Quality control samples and sampling shipping and preservation will be in accordance with the QAPP.

Sediment samples will be collected after the collection of co-located surface water samples to eliminate potential impact from disturbed sediments.  The sediment sample collection will be performed by excavating a sample using a decontaminated scoop.  Sampling locations where the surface water is deeper than a few inches, the loss of fine grain sediment material may be minimized by placing a PVC bucket with the bottom removed into the sediment to temporarily isolate the sediment sample extraction area from the flowing surface water.  The collected sediment will be homogenized by quartering and mixing in a bowl with a sampling scoop prior to transfer to sample containers.  Any sediment that remains after the containers have been filled will be returned to the location where the sample was collected.

Equipment

The following sediment sampling equipment may be needed:

Non-metallic sampling scoops and/or spoons

Non-metallic mixing bowl

PVC bucket with the bottom removed

Latex and/or nitrile gloves

Waders

COC forms

Field notebooks

Sample containers and preservatives

Cooler with ice

Sample location map

Sampling Procedures for Soil and Other Solids

Soil and/or former process material (e.g. slag) will be sampled with the procedures discussed below.  In general, two types of samples will be collected: surficial and boring samples.  Boring samples will be collected consistent with “Chapter 6:  Drilling and Subsurface Sampling” (OEPA 2007) and “Chapter 15:  Use of Direct Push Technologies for Soil and Ground Water Sampling” (OEPA 2005) of the OEPA Technical Guidance Manual.

Sampling equipment will be constructed of inert materials.  Proper equipment decontamination procedures will be followed to minimize the potential for cross-contamination.  Quality control samples and sampling shipping and preservation will be in accordance with the QAPP.

Equipment

The following sampling equipment may be needed:

Non-metallic sampling scoops and/or spoons

Non-metallic mixing bowls

Stainless steel knife

Latex and/or nitrile gloves

COC form

Field notebooks

Sample containers

Cooler with ice

Sample location map

Near-Surface Samples

This section describes the procedure that will be used to collect surface or near-surface soil and other solids (including slag).  Samples will be collected with decontaminated equipment (e.g., scoops, spoons, etc.) from the interval(s) prescribed in the RI/FS Workplan (after clearing away any leaves, sticks, etc.).  To the extent possible, the collected material will be homogenized by quartering and mixing in a bowl with a sampling scoop prior to transfer to sample containers.  Any material that remains after the containers have been filled will be returned to the location where the sample was collected.

Soil from Borings

This section describes the procedures and equipment that will be used to collect sub-surface soil samples.  Borings will be advanced using either a direct push technology (e.g., GeoProbe) or a hollow stem auger drilling rig with split spoon samples collected on a continuous basis through waste materials and soils adjacent to waste materials.  Regardless of the method, a core of soil from the desired depth, wrapped in a plastic sleeve from a GeoProbe or split spoon from hollow stem auger, will be removed from the ground and split open using a decontaminated knife.  To the extent possible, the borings will extend to the depth prescribed in the RI/FS Workplan.  Soil borings will be visually classified in the field in accordance with the Unified Soil Classification System (USCS).  Each boring is anticipated to yield the number of laboratory analytical samples prescribed in the RI/FS Workplan. Sample depths will be as specified in the RI/FS Workplan and field screening by visual inspection.

Decontamination and Investigation-Derived WasteS

The following sections present procedures for field decontamination of sampling equipment and procedures to handle spent wash and rinse waters from field decontamination and other potential investigation-derived waste (IDW).

Equipment

The following may be needed for the field decontamination procedures discussed below:

Tap water may be used from any potable water supply system

Deionized water

Distilled water

Detergent solution shall be a mixture of a laboratory detergent such as Liquinox® or Alconox® diluted in tap water.  Use of other detergents must be documented in the field notebooks

Solvent shall be pesticide-grade isopropyl alcohol.  Use of a solvent other than pesticide-grade isopropyl alcohol for equipment cleaning purposes must be justified and documented in field notebooks

Scrub brushes

Five-gallon buckets for collection of spent wash and rinse waters

Drill Rig and Drilling Equipment

Appropriate parts of the drill rigs and related downhole drilling and sampling equipment used at the Site will be decontaminated prior to use at the Site, between each borehole location, and prior to the equipment leaving the Site.

Decontamination of drill rigs and all downhole drilling equipment (non-sampling equipment) will be conducted at a decontamination pad located at the Site.  Decontamination will include steam cleaning and manual scrubbing, as necessary, to remove any visible contamination.

Decontamination of sampling equipment (i.e., split-spoon soil sampling devices) will be decontaminated as follows:

- Brush clean to remove obvious particulates matter

Brush with tap water and detergent solution

Rinse with tap water

Equipment

The following additional equipment will be needed to decontaminate a drill rig and drilling equipment:

Steam cleaner

Decontamination pad

Water Quality Meters and Water Level Meters

Prior to use at each individual sampling location, the water quality meter and water level meter will be rinsed with distilled or deionized water.  If the probes appear discolored, a mild detergent solution may be used to gently wash the probes.  The meters should be thoroughly rinsed with distilled or deionized water after using a detergent solution, and dried with clean paper towels or permitted to air dry.

Groundwater Sampling Pumps

Decontamination procedures listed below are intended for use by field personnel for cleaning non-dedicated groundwater sampling pumps:

- Use scrub brush or paper tower with detergent solution on portions of the cable spool that enter the water.

Rinse pump and spool with tap water.

Place short piece of discharge tubing on pump and place pump into Tube A.  Place the tubing discharge back into tube A so that the pump will recirculate the water.

Fill the tube with tap water and a small amount of detergent solution.

Activate pump for at least 1 minute occasionally move the pump up and down within the tube.

While pumping, remove the discharge tubing and direct discharge to a waste bucket.

Turn-off pump after flow stops.  Remove pump, dump out detergent wash from Tube A, then replace pump into Tube A.

Fill Tube A with tap water and place the tubing discharge back into tube A so that the pump will recirculate the water.

Activate pump for at least 30 seconds occasionally move the pump up and down within the tube.

While pumping, remove the discharge tubing and direct discharge to a waste bucket.

Turn-off pump after flow stops.  Remove pump, dump out tap water rinse from tube A, then replace pump into Tube A.

Repeat the tap water rinse three times.

Thoroughly rinse pump with deionized or distilled water.

Place pump in Tube B.  Only a clean pump and deionized or distilled water are to be placed in Tube B.

Fill Tube B with deionized or distilled water and direct discharge tubing into waste bucket.

When flow ceases, stop pump.  Remove pump and dump out water from Tube B.

Remove pump and disconnect discharge tubing.  Place pump and cable spool into a clean bag.

Equipment

The following additional equipment will be needed to decontaminate a groundwater sampling pump:

Two 2-inch diameter PVC tubes (“decon tubes”) approximately 2 feet long with one end capped.

Teflon-lined discharge tubing, approximately 3 feet long.

Other Sampling Equipment

The field decontamination procedures listed in this section are intended for use by field personnel for cleaning all other commonly used sampling equipment (e.g., mixing bowl, sampling spoon, knife, etc.) other than equipment previously mentioned.

- Clean with tap water and detergent, using a brush to remove obvious particulate matter and surface films.

- Rinse thoroughly with tap water.

- Rinse thoroughly at least three times with deionized or distilled water.

- Allow equipment to completely air dry.

- Wrap equipment in aluminum foil and/or place in plastic bags.

Investigation-Derived Waste

Investigation-derived waste (IDW), such as borehole cuttings, decontamination/well development/well purging water, and personal protective equipment, may be generated during the project.  All such IDW must be managed and disposed in accordance with applicable regulations.

Borehole Cuttings

All excess soil generated from drilling activities will be placed on the ground in vicinity of the sampling location, and raked to approximate grade level.

Water

All monitoring well development and purge water, decontamination fluids, detergent solution, and rinse water from equipment decontamination will be containerized in an above-ground tank until the ground water samples are analyzed.  Based on the results of the groundwater analysis, it will be determined if it is acceptable to discharge this purge water directly to the ground surface, away from surface water bodies or if the water will require off-Site disposal.

Personal Protective Equipment and Disposable Sampling Equipment

All personal protective equipment and disposable sampling equipment will be containerized in 55-gallon drums and temporarily stored at the Site.  Disposal will occur during final remedial action in accordance with the selected remedy.

References

Golder Associates Inc. (Golder) 2012.  Quality Assurance Project Plan for the Remedial Investigation.  Former Satralloy Site, Jefferson County, Ohio.  August 3.

Ohio Environmental Protection Agency (OEPA) 1995.  Chapter 7:  Monitoring Well Design and Installation, Technical Guidance Manual for Hydrogeologic Investigations and Ground Water Monitoring.

OEPA. 2001.  Sediment and Sampling Guide and Methodologies (2nd edition).  November.

OEPA. 2004.  Chapter 8:  Monitoring Well Development, Technical Guidance Manual for Hydrogeologic Investigations and Ground Water Monitoring.

OEPA. 2005.  Chapter 15:  Use of Direct Push Technologies for Soil and Ground Water Sampling, Technical Guidance Manual for Hydrogeologic Investigations and Ground Water Monitoring.

OEPA. 2006.  Chapter 10:  Ground Water Sampling, Technical Guidance Manual for Hydrogeologic Investigations and Ground Water Monitoring.

OEPA. 2007.  Chapter 6:  Drilling and Subsurface Sampling, Technical Guidance Manual for Hydrogeologic Investigations and Ground Water Monitoring.

ATTACHMENT H1

FISHING SAMPLING PROCEDURES

ATTACHMENT H2

FIELD PROCESSING AND HANDLING PROCEDURES FOR FISH SAMPLES

Golder Associates Inc.

18300 NE Union Hill Road, Suite 200

Redmond, WA  98052 USA

Tel:  (425) 883-0777

Fax:  (425) 882-5498